# Automation of NDOT Concrete Mix Design Excel

Prepared by:

Jacob Oetken, Anicet Atikpohou, Abbas Bashir, and Michael Albracht

Prepared For:

Nebraska Department of Transportation (NDOT)

Course: CIVE202 – Spring 2026
Instructor: Dr. Kaycie Lane

Table of Contents

Introduction and Background	3

Definitions	3

Scope of Work and Objectives	4

Translate NDOT Excel Logic into Python	4

Implement Sequential User Inputs	4

Generate Final Mix Design Output	4

Four Concrete Mix Scenarios	4

Prepare Deliverables	4

Methods	5

Translate NDOT Excel Logic into Python	5

Implement Sequential User Inputs	5

Mix Design Calculation and Aggregate Balancing	5

Generate Final Mix Design Output	5

Prepare and Evaluate Four Concrete Mix Scenarios	6

Known Shortfalls	6

Results	7

Code Output Examples	8

Conclusion	9

References	10

# Introduction and Background

The Nebraska Department of Transportation (NDOT) requested an assessment of whether its current Excel-based concrete mix design workflow could be converted into a structured Python system. The project aims to replicate the original “Mix Design” worksheet's logic while enhancing transparency, consistency, and repeatability. The new tool must generate a complete weight summary for one cubic yard of concrete based on user-defined material properties and design parameters.

A Python-based implementation secures calculation logic within defined functions, reducing the risk of unintended formula changes and improving the traceability of assumptions. A sequential input structure and a consistent volumetric balancing procedure are needed to standardize the mix design process and enable the repeatable evaluation of multiple concrete scenarios.

## Definitions

Mid-Range Water Reducer - MRWR: A chemical admixture for concrete that decreases the required mixing water while maintaining adequate workability. MRWR is intermediate between conventional water reducers and superplasticizers (ASTM International, 2019). It facilitates improved consolidation and finishing without rendering the mix excessively fluid. MRWR is commonly applied where moderate slump and satisfactory strength development are required, such as in footings, walls, and general structural members (UniSpec II, 2025).

High-Range Water Reducer - HRWR: Typically called a superplasticizer, which is a chemical concrete admixture that significantly reduces the water content while increasing flowability. HRWR enables high-strength and high-workability concrete simultaneously (American Concrete Institute, 2018; ASTM International, 2019). It is especially useful when concrete must flow easily into complex forms or be placed quickly without sacrificing mechanical performance.

Type IP Cement: Under ASTM C595, Type IP cement is a blended hydraulic cement consisting of portland cement interground with a specified percentage of pozzolanic material (ASTM International, 2023). The pozzolan content typically ranges from approximately 15–40% of the mixture. Pozzolanic materials commonly include fly ash, calcined clay, volcanic ash, or silica-rich natural materials. In practical terms, Type IP cement is designed to improve durability compared to ordinary portland cement through the addition of pozzolans, which create a tighter, denser internal structure.

## Scope of Work and Objectives

### Translate NDOT Excel Logic into Python

The Python environment needs to replicate all calculation procedures in the NDOT mix design Excel file. The result should preserve mathematical relationships and constants within the spreadsheet. The objective is to ensure that the Python model produces the same output for the same input.

### Implement Sequential User Inputs

The Python model should incorporate a structured input that corresponds to the workflow in the original Excel. A user must be able to enter all required material properties and parameters in a logical format. Required inputs include cementitious material quantities, water–cement ratio, air content, aggregate distributions, and material specific gravities. The objective is to ensure consistent data entry and eliminate missing parameters before calculation.

### Generate Final Mix Design Output

The Python model needs to produce a clearly formatted weight summary for a one-cubic-yard batch of concrete. The output should consist of the following values: Water weight, Cement weight, SCM, Fine Aggregate weight, Coarse Aggregate weight, and any extra materials included. The final output must reflect complete volumetric balancing for a one-cubic-yard batch and be suitable for engineering review.

### Four Concrete Mix Scenarios

The Python model must be evaluated using four concrete mix scenarios based on research, representing varied applications, strength classifications, placement methods, and admixture types. Each scenario must be processed through the model to verify computational consistency and dependability over different mix configurations.

### Prepare Deliverables

The following documents and documentation should be developed and submitted with the final project:

#### Gantt Chart

A project schedule identifying sequencing, dependencies, duration, and milestones.

#### Scope of work

A formal/professional document outlining project objectives, required tasks, and deliverables.

#### GitHub Repository

A centralized, shareable repository containing the Python program, a README document explaining the use of the program, and any supporting documentation.

# Methods

### Translate NDOT Excel Logic into Python

The NDOT mix design logic was replicated in a Python environment using the absolute-volume balancing approach. The target batch size was one cubic yard (27 ft³). Initially, the program defines constants for the batch size and unit weight of water (62.4 lb./ft³). Cementitious material was converted from weight to absolute volumes using its specific gravity as well as the unit weight of water. Entrained air is displayed as a percentage of the batch volume. The water content is calculated directly from the total cement weight and the specified water-to-cementitious ratio. Once the volumes for cement, SCM, water, and air are determined, the remaining volume is distributed to the total aggregate volume

### Implement Sequential User Inputs

User inputs are collected within the program in the same order as they appear in the Excel workbook. The program first prompts the user for client and project information to link the concrete to a specific job or task. The next item requested is the cementitious material weights (cement, fly ash, silica fume, and other SCM), followed by the target water-to-cementitious ratio and target air content. The user must then provide the desired aggregate distribution percentages (fine, coarse, other). To convert from weight to absolute volume for the calculations, the user must enter the specific gravity for each material category. These inputs are passed to a single mix design function to ensure no variation between runs or tests. The same workflow is used for both user-defined designs and predefined scenarios.

### Mix Design Calculation and Aggregate Balancing

Within the reusable mix design function, cement, SCM water, and air are converted into volumes (ft³). The total aggregate volume is then computed from the remaining volume required to make the batch total 27 ft³, once the volumes for all other components have been determined. Aggregate weights are then found by distributing the total aggregate volume among fine, coarse, and other aggregate components using the specified percentages and specific gravities. This produces final batch weights for each material component for one cubic yard of concrete.

### Generate Final Mix Design Output

Results are reported as a formatted weight summary for one cubic yard of concrete, listing cementitious materials, aggregate weights by category, and water weight. For ease of interpretation, the program computes and prints the total batch mass (lb./yd³). In addition to a user-input design case, four researched mix scenarios are evaluated by assigning fixed input values equal to the user-provided values and passing them through the same mix design function. This approach delivers a consistent, verifiable calculation process, enabling comparison of batch quantities across a variety of mix configurations.

### Prepare and Evaluate Four Concrete Mix Scenarios

#### Mix 1 (LF4500 W/MRWR)

This mixture represents structural footing concrete that was designed for the Eppley Terminal Entrance Project. This concrete is designed for 4500 psi. It is intended for a below-grade application that requires extra durability. The mid-range water reducer (MRWR) improves workability without significantly increasing slump or reducing strength (Sika Corporation, 2020). This mix balances strength and durability for structural footings where excessive flow is not required, but consolidation is critical.

#### Mix 2 (340MYL7A - OPW4000)

This mix is essentially Omaha’s standard concrete with the addition of a high-range water-reducing admixture (HRWR). The HRWR significantly improves flow without increasing water content (Zhang et al., 2023). This is beneficial for hand placement, complex forming, and improved finishing operations while allowing for a low water/cement ratio for strength and durability (4000 psi).

#### Mix 3 (340MYL3 - OPW4000)

This mix again is the standard concrete for work within the public right-of-way in Omaha. However, the slump range is much lower, and no superplasticizers are added. Rather than hand placing, this concrete will be placed with a slip form paver. These pavers require lower slump concrete to maintain edge stability and shape retention. Excessive flowability would cause edge collapse (Concrete Masonry & Hardscapes Association, 2022). Therefore, this mix is specific to machine placement and retained durability (4000 psi).

#### Mix 4 (345C2L3B - L6 1/2 PAE (IPF) – P-610 Equivalent)

This mix represents a 4000 psi FAA-classified structural pavement mix using a blended Type IP cement. This mix focuses on durability and long-term performance, particularly sulfate resistance and alkali-silica reaction mitigation via blended cement (Holcim US, n.d.). The aggregate balance supports finishing quality and pavement surface performance.

### Known Shortfalls

- The program, as it sits, does not currently check that the aggregate (fine, coarse, other) totals 100%.

- There is no check that the total_agg_volume remains positive.

- If a specific gravity is entered as zero or left blank for any material, the program will encounter a divide-by-zero error during volume calculations and terminate execution.

- Additionally, admixtures such as MRWR and HRWR are discussed in concept but are not modeled in the calculations. The program, therefore, represents a volumetric proof-of-concept rather than a ready-to-go batching system.

# Results

The Python model was first evaluated using manually entered user input values corresponding to Mix Design 1. The same material quantities and design parameters were then handled using the predefined Mix 1 scenario embedded within the code. Next, those values were compared to the known mix results. For this situation, the Python model calculated a mass of 3790.93 lb/yd³. This value was off by approximately 30 pounds, representing a deviation of around 0.8% from the total batch weight. Given that most of the batch weight comes from both aggregate types (~2900lbs.), a 30lb deviation in aggregate mass alone is just 1%.

In practice, aggregate batching relies heavily on moisture levels within the aggregates. If the aggregate is not at SSD (saturated-surface-dry), the batched water and aggregate masses shift. When aggregate is wet, it adds free water, increasing the measured aggregate weight, whereas dry aggregate absorbs water, therefore decreasing effective free water. Another piece, smaller but still contributing a few pounds, would be the failure to incorporate admixtures. The mix calls for 6oz per 100lbs of cementitious material, which works out to under three lbs. for this mixture.

As shown in (Figure 1) and (Figure 2), both methods produced identical output values for all material components, including cement, fly ash, aggregates, water, and total batch mass. This confirms that the reusable mix_design() function performs consistent calculations regardless of whether the inputs are provided by the user or via predefined scenario variables. This verifies that translating the Excel-based logic into Python preserved mathematical equivalence across identical input conditions.

Mix Designs two through four were evaluated using the same mix_design() function. In each case, the model generated volumetrically balanced batch weights for one cubic yard without producing non-physical results. Differences between the mixes reflected expected engineering behavior:

- Lower water/cementitious ratios in Mix 3 vs. Mix 2 resulted in reduced water weight, leading to increased aggregate volume.

- A higher fine aggregate percentage (Mix 4) shifted the aggregate weight distribution toward fine aggregate while reducing coarse aggregate mass.

- Variations in air content (5.6–7.5%) directly influenced total aggregate volume, consistent with the absolute volume method.

Across all four scenarios, material values adjusted as expected for water/cementitious ratio, air content, and aggregate distribution, indicating consistent implementation of the initial Excel volumetric calculations.

### Code Output Examples

Figure 1: User Input Mixture Weight Summary

Figure 2: Hard-Coded Mix Design 1 Weight Summary

# Conclusion

This project successfully translated the NDOT Excel-based concrete mix design workbook into a clearly structured Python model using an absolute volume calculation for one cubic yard of concrete. The model produced consistent outputs across manual and predefined scenarios, confirming that the original logic in the Excel file was preserved.

Overall comparisons to known batch data showed a variation of approximately 0.8%, which is within reasonable practical tolerance and most likely attributable to aggregate moisture variation, batching tolerances, and minor excluded mass components such as admixtures.

In all evaluated scenarios, the program responded logically to changes in material variables, indicating consistent volumetric balance in the calculations. While the model acts as a proof-of-concept rather than a fully calibrated batching program, it provides a transparent, repeatable framework suitable for further development into a more solid design platform.

# References

American Concrete Institute. (2018). Guide to residential concrete (ACI 332.1R-18). https://www.concrete.org/Portals/0/Files/PDF/Previews/332.1R-18_preview.pdf

ASTM International. (2019). ASTM C494/C494M-19: Standard specification for chemical admixtures for concrete. ASTM International.

ASTM International. (2023). ASTM C595/C595M-23: Standard specification for blended hydraulic cements. ASTM International. https://doi.org/10.1520/C0595_C0595M-23

Concrete Masonry & Hardscapes Association. (2022). Edge restraints for interlocking concrete pavements. https://www.masonryandhardscapes.org/resource/pav-tec-003-22/

Holcim US. (n.d.). FortiCem® Portland-pozzolan blended cement. https://www.holcim.us/forticem

Sika Corporation. (2020). Mid-range water reducer. https://usa.sika.com/en/construction/concrete/concrete-admixtures/water-reduction/mid-range-water-reducer.html

Sika Corporation. (n.d.). Sikament®-475 | Mid-range water reducers. https://usa.sika.com/en/construction-products/concrete/concrete-admixtures/water-reduction/mid-range-water-reducers/sikament-475.html

UniSpec II. (2025). Structural concrete and exterior concrete slabs (Section 03310). JRJ Projects. https://jrj-projects.gswconst.biz/BillyShipley/2979%20Ft%20Worth%20NHM_/Bid%20Spec/Specifications/03310%20-%20structural%20concrete%20and%20exterior%20concrete%20slabs%20-%202979.pdf

Zhang, X., Li, Y., & Wang, Z. (2023). Utilization of high-range water reducing admixture having air-entraining agents in cementitious systems. Journal of Building Engineering, 64, 105565. https://doi.org/10.1016/j.jobe.2022.105565